Teacher answer booklet

Lesson 1:

Architecture of the CPU

OBJECTIVES:

- To understand the purpose of the CPU and actions that occur within the fetch-execute cycle.

- To understand the function of common CPU components.

- To understand the purpose of each register used by the CPU within the Von Neumann architecture.

STARTER ACTIVITY: WORDSEARCH

Find the key vocabulary terms in the wordsearch below.

Play Online - Easy Word Search Maker

TASK 1: VON NEUMANN ARCHITECTURE

- In the diagram below, label each component and register that is part of the Von Neumann architecture.

Input device

Output device

ACC

MDR

MAR

PC

Memory Unit

Registers

Arithmetic Logic Unit

Control Unit

- Using the acronyms, identify what each component or register stands for.

| CU | ALU | PC | MAR | MDR | ACC |
| --- | --- | --- | --- | --- | --- |
| Control Unit | Arithmetic Logic Unit | Program Counter | Memory Address Register | Memory Data Register | Accumulator |

TASK 2: SYSTEMS ARCHITECTURE

- Match each register to the correct purpose shown below.

| Program Counter |  | - Stores the results of calculations carried out by the ALU. It stores the calculation values |
| --- | --- | --- |
| Memory Address Register |  | - Holds the address in memory that the CPU needs to access. It stores the address |
| Memory Data Register |  | - Holds the data or instruction that has been fetched from memory or is about to be written to memory |
| Accumulator |  | - Stores the address of the next instruction to be fetched. It stores the address |

- Fill in the missing gaps to reveal the common components of the CPU.

Cache speeds up CPU by reducing access to RAM by storing frequently used instructions. Arithmetic Logic Unit performs all the mathematical calculations and logical decisions. Control Unit manages and co-ordinates tasks with the CPU.

- In the diagram below, identify what happens to the instruction at each stage of the fetch-decode-execute cycle.

TASK 3: VON NEUMANN ARCHITECTURE

Activity A

Use a tick or a cross to identify the similarities and differences between each register.

Activity B

The table below shows examples of storing data and storing addresses. Tick one box per row to identify what the examples are showing.

|  | Storing data | Storing an address |
| --- | --- | --- |
| A character like A |  |  |
| A calculation results such as 101010 (binary for 42) |  |  |
| Memory location |  |  |
| RAM location where a variable is stored |  |  |
| The number 42 |  |  |

Activity C

What is the difference between storing an address and storing data?

Storing data means storing the actual value you want to use. Storing an address means storing the location in memory where data is stored, not the data itself.

TASK 4: THE ROLE OF COMPONENTS IN THE FETCH-EXECUTE CYCLE

In the table below, tick () one or more boxes per row to identify which part of the fetch-decode-execute cycle each component contributes to. In the final column, you need to explain why, you can give examples of what tasks might be performed.

| Component | Fetch | Decode | Execute | Justification |
| --- | --- | --- | --- | --- |
| Arithmetic Logic Unit |  |  |  | During the cycle, the ALU is mainly active in the execute stage, when the CPU carries out the instruction. This could include: - Arithmetic operations (addition, subtraction, multiplication, division) - Logical operations (AND, OR, NOT, XOR) - Comparisons (greater than, equal to, etc.) - Bit shifting Students may not know any of these at this stage. |
| Cache |  |  |  | During the fetch stage, the CPU checks cache first before going to RAM, which speeds up processing. |
| Control Unit |  |  |  | The Control Unit directs how data moves around the CPU and tells components what to do. It manages: - Fetching instructions from memory - Decoding instructions - Sending control signals to other components - Timing of operations - Movement of data between registers, ALU, cache, and memory |

EXAM STYLE QUESTION

Command word alert!

Describe means to give a detailed account or picture of a situation, event, pattern or process.

Describe the role of one register in the Von Neumann Architecture. (3 marks)

Mark Scheme ✅ 1 mark = naming the register (its name) ✅ 2 marks = bolded descriptive points ✅ Written using the command word describe Model Answers 1) Program Counter (PC) — 3 marks The Program Counter (PC) stores the address of the next instruction to be fetched from memory. During the fetch stage, this address is copied to the MAR so the CPU can locate the correct instruction. 2) Memory Address Register (MAR) — 3 marks The Memory Address Register (MAR) holds the address of the memory location the CPU wants to access. It receives this address from the PC and sends the address to memory so the correct instruction. 3) Memory Data Register (MDR) — 3 marks The Memory Data Register (MDR) stores the data or instruction that has been transferred from memory. After memory is accessed using the MAR, the MDR holds the fetched instruction before passing it to other CPU components for decoding or processing. 4) Accumulator (ACC) — 3 marks The Accumulator (ACC) stores the results of calculations performed by the ALU. During execution, the ALU places intermediate or final results in the ACC so they can be used in subsequent operations or written back to memory.

STRETCH & CHALLENGE

To develop a deeper understanding of the roles of the existing Von Neumann registers (PC, MAR, MDR, ACC, CIR) and how they work together during the fetch–decode–execute cycle.

Activity A: Deep thinking questions

1. Why does the PC store an address rather than the instruction itself?
The PC stores an address because instructions are stored in RAM, not inside the PC. By holding the address, the CPU can fetch instructions from memory one at a time and execute programs of any size.

2. Why are both the MAR and MDR needed — why can’t one register do both jobs?
The MAR holds an address, while the MDR holds data or instructions. These must be separate because the CPU often needs to manage both simultaneously, and addresses and data travel along different buses (address bus and data bus).

3. How does the ACC help reduce the need to access RAM?
The ACC stores intermediate results of calculations performed by the ALU. By keeping results inside the CPU, the processor does not need to repeatedly write to and read from slower RAM, which improves performance.

4. Which register is most critical for ensuring instructions execute in the correct order? Justify your answer.
The PC is most critical because it keeps track of the address of the next instruction. By incrementing after each fetch, it ensures instructions are executed sequentially unless a jump or branch occurs.

Activity B: Fault-Finding Challenge

| Register Fails | What would happen to the computer? |
| --- | --- |
| PC | The CPU would not know which instruction comes next, so programs would stop or behave unpredictably. |
| MAR | The CPU could not select memory locations, so it would be unable to fetch instructions or data from RAM. |
| MDR | Data and instructions could not be transferred between memory and the CPU, preventing execution. |
| ACC | The CPU could not store intermediate calculation results, making arithmetic operations inefficient or impossible. |

Activity C

Why do registers make the CPU faster?

Registers are much faster than RAM because they are located inside the CPU. For example, the ACC stores calculation results so the processor can reuse them immediately without accessing memory, and the PC keeps track of the next instruction without needing to search RAM. This reduces memory access time and allows instructions to be executed more quickly.

Terms of Use: Copyright Notice

© SIMPLY TEACH 2026

These Terms of Use govern the use of any material, content, or intellectual property provided by Simply Teach. By accessing or using the Material provided by Simply Teach, you agree to be bound by these Terms of Use.

- Copyright Notice: All Material provided by Simply Teach is protected by copyright laws. All rights are reserved by Simply Teach.

- Non-Redistribution: You may not redistribute, reproduce, republish, transmit, distribute, or otherwise use the Material provided by Simply Teach without prior written permission from Simply Teach.

- No Public Domain Usage: The Material provided by Simply Teach is not released into the public domain. You may not use, modify, or distribute the Material in any way that would place it in the public domain.

- Intellectual Property Rights: All intellectual property rights in the Material, including but not limited to copyrights, trademarks, patents, and trade secrets, belong to Simply Teach. You agree not to infringe upon these rights in any way.

Contact Information: If you have any questions about these Terms of Use, please contact Simply Teach at info@mysimplyteach.co.uk or visit our Facebook page.

By accessing or using the Material provided by Simply Teach, you acknowledge that you have read, understood, and agree to be bound by these Terms of Use. If you do not agree to these terms, you may not access or use the Material.