1.1.1 SYSTEMS ARCHITECTURE – KEY VOCABULARY

🧠 1.1.1 Architecture of the CPU

| Keyword | Definition | Example Sentence |
| --- | --- | --- |
| CPU (Central Processing Unit) | The part of the computer that processes instructions and controls operations | The CPU carries out instructions to run programs on the computer. |
| Fetch-Execute Cycle | The process where the CPU fetches, decodes, and executes instructions | During the fetch-execute cycle, the CPU retrieves and processes instructions from memory. |
| Fetch | The stage where an instruction is copied from memory | In the fetch stage, the CPU gets the next instruction from RAM. |
| Decode | The stage where the instruction is interpreted | The decode stage allows the CPU to understand what the instruction means. |
| Execute | The stage where the instruction is carried out | In the execute stage, the CPU performs the required operation. |
| ALU (Arithmetic Logic Unit) | Performs calculations and logical operations | The ALU is used to carry out addition and comparisons. |
| Control Unit (CU) | Controls the flow of data and instructions in the CPU | The Control Unit manages the fetch-execute cycle. |
| Cache | Small, fast memory close to the CPU storing frequently used data | Cache improves performance by storing commonly used instructions. |
| Registers | Very small, fast storage locations inside the CPU | Registers store data temporarily during processing. |
| MAR (Memory Address Register) | Stores the address of data or instructions to be fetched | The MAR holds the address of the next instruction. |
| MDR (Memory Data Register) | Stores the data being transferred to/from memory | The MDR stores the instruction fetched from memory. |
| Program Counter (PC) | Holds the address of the next instruction to be executed | The Program Counter increases after each instruction. |
| Accumulator | Stores the result of calculations from the ALU | The accumulator holds the result of an addition. |
| Data | Actual values being processed | The MDR stores data that the CPU needs to process. |
| Address | A location in memory where data is stored | The MAR stores the address of the required data. |

⚡ 1.1.2 CPU Performance

| Keyword | Definition | Example Sentence |
| --- | --- | --- |
| Clock Speed | Number of cycles the CPU can perform per second (GHz) | A higher clock speed means the CPU can process more instructions per second. |
| Cache Size | The amount of cache memory available | Increasing cache size can improve performance by reducing memory access time. |
| Number of Cores | Number of processing units in a CPU | A CPU with more cores can run multiple tasks at the same time. |
| Performance | How fast and efficiently a CPU works | CPU performance improves with higher clock speed and more cores. |
| Multicore Processing | Using multiple cores to run tasks simultaneously | Multicore processing allows better multitasking. |

🤖 1.1.3 Embedded Systems

| Keyword | Definition | Example Sentence |
| --- | --- | --- |
| Embedded System | A computer system built into a device to perform a specific task | A washing machine uses an embedded system to control cycles. |
| Dedicated Function | Designed to perform one specific task | Embedded systems have a dedicated function such as controlling a microwave. |
| Real-Time Processing | Responds immediately to inputs | A car’s braking system uses real-time processing. |
| Microcontroller | A small processor used in embedded systems | A microcontroller is used inside a thermostat. |
| Sensors | Devices that detect changes in the environment | Sensors in an embedded system detect temperature. |
| Firmware | Software programmed into hardware | Firmware controls how an embedded system operates. |